- Hollow Shaft Encoder – Ø68 mm

- Hollow Shaft: Ø 3/4 inch, Ø 1 inch

- Resolutions up to 10.000 ppr

- Delivered with cable or MIL connector

- ATEX, IECEx

| Mechanical Specifications |  |
| --- | --- |
| Material: | Housing: Aluminum Cap: Aluminum Shaft: Stainless Steel (AISI 316) |
| Weight: | Encoder: Approx. 380 gr (13.4 oz) |
| Bearing Life: | > 1.9 x 1010 revolutions at rated load |
| Shaft Speed: | max. 2000 rpm |
| Starting Torque: | < 0.01 Nm (1.4 oz-in) at 25° C |
| Mass Moment of Inertia: | 440 gcm² (6.23 x 10-3 oz-in-sec²) |
| Shaft Loads: | Axial: 100 N (22,5 lbs) max. Radial: 100 N (22,5 lbs) max. |

| Electrical Specifications |  |
| --- | --- |
| Code: | Incremental |
| Resolution: | 1 to 10,000 ppr (pulses per revolution) |
| Supply Voltage: | 4.5 Vdc min. to 30 Vdc max. ** (at 45 mA max. - no load) |
| Output Voltage: | Low: 500 mV max. at 10 mA High: (Vsup – 0.6) at -10 mA (Vsup – 1.3) at -25 mA |
| Output Current: | 30 mA max. load ** per output channel |
| Frequency Response: | 300 kHz max. ** |
| Output Format: | Two channel (A, B) quadrature with Index (Z) and optional complementary (A-, B-, Z-) outputs |
| Phase Sense: | A leads B counterclockwise (CCW) from the shaft end of the encoder |
| Index: | Gated with Channel B low |
| Accuracy: | +/- 0.8 arc-min. |
| Outputs: | ASIC Push-pull and Differential OL7272 Push-pull and Differential Line Driver 26C31 Differential Line Driver 5V output (with 5V input) |
| Electrical Protection: | Output short circuit protected. Reverse polarity protected for ASIC outputs only. (not for OL7272 or 26C31 outputs) |
| Noise Immunity: | EN61000-6-2: 2005 (industrial environments) Electromagnetic compatibility (EMC) EN 61000-6-3: 2007 (residential, commercial, and light-industrial environments) for Electromagnetic compatibility (EMC) |

**= It is recommended user not to combine max. Value for all 3 parameters

| Environmental Specifications |  |
| --- | --- |
| Operating Temp.: | -40° to +65° C |
| Storage Temp.: | -40°C to +85°C |
| Shock: | 100g @ 11 ms |
| Vibration: | 10g @ 10-2000 Hz |
| Bump: | 10g @ 16 ms (1000 x 3 axis) |
| Humidity: | 98 % RH without condensation |
| Enclosure Rating: | IP 64 / Nema 3 (approx.) – option IP 65 / Nema 4 (approx.) – option IP 66 / Nema 6 (approx.) – option IP 67 / Nema 6 (approx.) – option |

| Connection Options |  |
| --- | --- |
| Cable: | MUD resistant (NEK 606) & halogen free, shielded, OD Ø6.6 +/- 0.2 mm 4x2x0,14 mm2, 26 AWG, 4 twisted pairs; shielded - Table 2 |
| Connectors: | 10-pin MIL radial 10-pin MIL Bayonet radial Table 3 |

SCH68BEX

| Certifications |  |
| --- | --- |
| ATEX: | Certificate No.: ITS I24 ATEX39692X II 3 G Ex ec IIC T4 Gc -40°C ≤ Tamb ≤ +65°C* |
| IECEx: | Certificate No.: IECEx ITS 24.0001X Ex ec IIC T4 Gc -40°C ≤ Tamb ≤ +65°C* |
| EAC Ex: | N/A |

* Upper ambient can be any value between +20°C and +65°C to meet customer requirements.

Table 1. Disk Resolutions (pulses per revolution)

Output waveform

Other options on request

Pulses per revolution,

min. 1 – max. 10.000

Channel tolerance                   180 e°  +/-  36 e°

Phase difference tolerance       90 e°  +/-  18 e°

Z channel tolerance                 180 e°  +/-  18 e°

* Operating temperature:   -20° C to 50° C

mm (inches)

mm (inches)

MIL Bayonet connector

MIL connector

| Mechanical dimensions |  |
| --- | --- |
| Mechanical Dimensions |  |

|  | 10-pin MIL Bayonet |  | 10-pin Mil |  |
| --- | --- | --- | --- | --- |
|  | Differential Output | Optional Differential Output * | Differential Output | Optional Differential Output * |
| Pin | Channel | Channel | Channel | Channel |
| A | A | A | A | A |
| B | B | B | B | B |
| C | Z | A - | Z | A - |
| D | Vsup | Vsup | Vsup | Vsup |
| E | NC | B - | NC | B - |
| F | GND | GND | GND | GND |
| G | Shield | Shield | Shield | Shield |
| H | A - | NC | A - | NC |
| I | - | - | B - | NC |
| J | B - | NC | Z - | NC |
| K | Z - | NC | - | - |

| Table 3. Output Terminations – MIL Connectors |  |
| --- | --- |
| Mechanical Dimensions |  |

GND = Circuit Ground    Shield = Case Ground

|  | Standard Cable |  |
| --- | --- | --- |
|  | Standard Output | Differential Output |
| Channel | Wire Color | Wire Color |
| A | Pink | Pink |
| A - | Gray* | Gray |
| B | Green | Green |
| B - | Yellow* | Yellow |
| Z | White | White |
| Z - | Brown* | Brown |
| Vsup | Red | Red |
| GND | Blue | Blue |

*  Gray, Yellow, and Brown are internally connected as Circuit Ground

GND = Circuit Ground

Shield connected to Case Ground

Cable:  Shielded, 4x2x0,14 mm2, 26 AWG, 4 twisted pairs;

Yellow cable shoe:  AWG 12-10

| Table 2. Cable Output |  |
| --- | --- |
| Mechanical Dimensions |  |

Table 3. Cable Output

mm (inches)

Standard Cable gland

Example:  SCH68BEX – 1024 – AL – D – 3/4 – 00 – 67 – 00 – B10 – A – S1

Ordering Code

| SCH68BEX | - |  | - | Al | - |  | - |  | - | 00 | - |  | - |  | - |  | - | A | - |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 |  | 2 |  | 3 |  | 4 |  | 5 |  | 6 |  | 7 |  | 8 |  | 9 |  | 10 |

| 1. Pulse per revolutions See Table 1 on page 2 | 2. Material Aluminum .................................. Al |  |  | 3. Output Standard ........................................... N Standard – open Collector NPN ...... NON Standard – open Collector PNP........ NOP Differential ....................................... D 26C31 Line Driver 5V only ............. L OL 7272 Line Driver ....................... M |
| --- | --- | --- | --- | --- |
| 4 & 5 Shaft dimensions 3/4 inch x 50 mm 1 inch x 50 mm | 4. 3/4 1/1 | x x | 5. 00 00 | 6. IP Rating IP 64 ................................................. 64 IP 65 ................................................. 65 IP 66 ................................................. 66 IP 67 ................................................. 67 |
| 7. Cable Length Standard cable is 1m............ 01 Specify length...................... XX No cable............................... 00 | 8. Cable & Connector Takeout Side Standard.......................... SS MIL – 10-pin........................ M10* MIL Bayonet – 10-pin........... B10** *part # MIL MS3102E-18-10 pin ** part # Bayonet MIL MS3112E-12-10P |  |  | 9. Flange Flange A .......................................... A |
| 10. Accessories 80131017 ........................ S1 80131067 ........................ S2 80132464 ........................ S3 80131062 ........................ S4 00232525 ........................ S5* *Only for encoder with MIL Bayonet conn. |  |  |  |  |